PROJECT 3 TOPOLOGY

incorrect IP address assignment error in the router 1

interface Ethernet1/0

ip address 10.0.0.1 255.0.0.0

interface Ethernet1/1

ip address 10.0.1.1 255.0.0.0

% 10.0.0.0 overlaps with Ethernet1/0

Ethernet1/1: incorrect IP address assignment

This error is because of your subnet mask.

What was Configured:

Ethernet1/0 → 10.0.0.1 255.0.0.0

255.0.0.0 is /8, which means the entire 10.0.0.0 – 10.255.255.255 range is one network.

So when entered second interface 1/1 :

Ethernet1/1 → 10.0.1.1 255.0.0.0

the router sees both interfaces as belonging to the same network: 10.0.0.0/8.

That's why we get error as % 10.0.0.0 overlaps with Ethernet1/0

Correct configuration

If you want two different networks, use /24:

interface Ethernet1/0

ip address 10.0.0.1 255.255.255.0

no shutdown

exit

interface Ethernet1/1

ip address 10.0.1.1 255.255.255.0

no shutdown

exit

Now the networks are:

| Interface | IP Address | Mask | Network |
| --- | --- | --- | --- |
| Ethernet1/0 | 10.0.0.1 | 255.255.255.0 (/24) | 10.0.0.0 |
| Ethernet1/1 | 10.0.1.1 | 255.255.255.0 (/24) | 10.0.1.0 |

These are two separate networks, so the router will accept them.

Complete static-routing configuration for Router 1 to Router 7.

Make all server networks communicate with each other, example:

Server 0 (10.0.0.10) → Server 8 (153.69.0.10)

Default interface ip configuration

First, keep this addressing table to Configure:

| Router | Interface | IP Address | Network | Connected to |
| --- | --- | --- | --- | --- |
| R1 | Eth1/0 | 10.0.0.1/24 | 10.0.0.0/24 | Server LAN |
| R1 | Eth1/1 | 10.0.1.1/24 | 10.0.1.0/24 | R2 |
| R2 | Eth1/2 | 10.0.1.2/24 | 10.0.1.0/24 | R1 |
| R2 | Eth1/1 | 10.0.2.1/24 | 10.0.2.0/24 | R3 |
| R2 | Eth1/3 | 10.0.6.1/24 | 10.0.6.0/24 | R7 |
| R2 | Eth1/0 | 200.10.156.1/24 | 200.10.156.0/24 | Server LAN |
| R3 | Eth1/2 | 10.0.2.2/24 | 10.0.2.0/24 | R2 |
| R3 | Eth1/0 | 10.0.3.1/24 | 10.0.3.0/24 | R4 |
| R3 | Eth1/1 | 10.0.4.1/24 | 10.0.4.0/24 | R5 |
| R4 | Eth1/1 | 10.0.3.2/24 | 10.0.3.0/24 | R3 |
| R4 | Eth1/0 | 182.46.0.1/24 | 182.46.0.0/24 | Server LAN |
| R5 | Eth1/0 | 10.0.4.2/24 | 10.0.4.0/24 | R3 |
| R5 | Eth1/1 | 10.0.5.1/24 | 10.0.5.0/24 | R6 |
| R6 | Eth1/0 | 10.0.5.2/24 | 10.0.5.0/24 | R5 |
| R6 | Eth1/1 | 135.82.0.1/24 | 135.82.0.0/24 | Server LAN |
| R7 | Eth1/0 | 10.0.6.2/24 | 10.0.6.0/24 | R2 |
| R7 | Eth1/1 | 153.69.0.1/24 | 153.69.0.0/24 | Server LAN |

Configure Router 1 static routes

directly connected routes:

10.0.0.0/24

10.0.1.0/24

Configure Router 1 static routes

ip route 10.0.2.0 255.255.255.0 10.0.1.2

ip route 10.0.3.0 255.255.255.0 10.0.1.2

ip route 10.0.4.0 255.255.255.0 10.0.1.2

ip route 10.0.5.0 255.255.255.0 10.0.1.2

ip route 10.0.6.0 255.255.255.0 10.0.1.2

ip route 182.46.0.0 255.255.255.0 10.0.1.2

ip route 135.82.0.0 255.255.255.0 10.0.1.2

ip route 153.69.0.0 255.255.255.0 10.0.1.2

ip route 200.10.156.0 255.255.255.0 10.0.1.2

Configure Router 2 static routes

directly connected routes:

10.0.1.0/24

10.0.2.0/24

10.0.6.0/24

200.10.156.0/24

Route to R1's LAN Next hop = R1 10.0.1.1

Routes toward R3 Next hop = R3 10.0.2.2

Route toward R7 Next hop = R7 10.0.6.2

Configure Router 2 static routes

R1 Server LAN

ip route 10.0.0.0 255.255.255.0 10.0.1.1

R4 side

ip route 10.0.3.0 255.255.255.0 10.0.2.2

ip route 182.46.0.0 255.255.255.0 10.0.2.2

R5/R6 side

ip route 10.0.4.0 255.255.255.0 10.0.2.2

ip route 10.0.5.0 255.255.255.0 10.0.2.2

ip route 135.82.0.0 255.255.255.0 10.0.2.2

R7 Server LAN

ip route 153.69.0.0 255.255.255.0 10.0.6.2

Configure Router 3 static routes

directly connected routes:

10.0.2.0/24

10.0.3.0/24

10.0.4.0/24

For R1/R2 side R2 = 10.0.2.1

For R4: R4 = 10.0.3.2

For R5: R5 = 10.0.4.2

Configure Router 3 static routes

! Toward R1/R2

ip route 10.0.0.0 255.255.255.0 10.0.2.1

ip route 10.0.1.0 255.255.255.0 10.0.2.1

ip route 10.0.6.0 255.255.255.0 10.0.2.1

ip route 200.10.156.0 255.255.255.0 10.0.2.1

ip route 153.69.0.0 255.255.255.0 10.0.2.1

! Toward R4

ip route 182.46.0.0 255.255.255.0 10.0.3.2

! Toward R5/R6

ip route 10.0.5.0 255.255.255.0 10.0.4.2

ip route 135.82.0.0 255.255.255.0 10.0.4.2

Configure Router 4 static routes

directly connected routes:

10.0.3.0/24

182.46.0.0/24

Everything else goes toward R3: R3 = 10.0.3.1

Configure Router 4 static routes

ip route 10.0.0.0 255.255.255.0 10.0.3.1

ip route 10.0.1.0 255.255.255.0 10.0.3.1

ip route 10.0.2.0 255.255.255.0 10.0.3.1

ip route 10.0.4.0 255.255.255.0 10.0.3.1

ip route 10.0.5.0 255.255.255.0 10.0.3.1

ip route 10.0.6.0 255.255.255.0 10.0.3.1

ip route 135.82.0.0 255.255.255.0 10.0.3.1

ip route 153.69.0.0 255.255.255.0 10.0.3.1

ip route 200.10.156.0 255.255.255.0 10.0.3.1

Configure Router 5 static routes

directly connected routes:

10.0.4.0/24

10.0.5.0/24

R3 = 10.0.4.1

R6 = 10.0.5.2

Configure Router 5 static routes

! Toward R3/R2/R1/R7

ip route 10.0.0.0 255.255.255.0 10.0.4.1

ip route 10.0.1.0 255.255.255.0 10.0.4.1

ip route 10.0.2.0 255.255.255.0 10.0.4.1

ip route 10.0.3.0 255.255.255.0 10.0.4.1

ip route 10.0.6.0 255.255.255.0 10.0.4.1

ip route 182.46.0.0 255.255.255.0 10.0.4.1

ip route 153.69.0.0 255.255.255.0 10.0.4.1

ip route 200.10.156.0 255.255.255.0 10.0.4.1

! Toward R6

ip route 135.82.0.0 255.255.255.0 10.0.5.2

Configure Router 6 static routes

directly connected routes:

10.0.5.0/24

135.82.0.0/24

Everything else goes to R5: R5 = 10.0.5.1

Configure Router 6 static routes

ip route 10.0.0.0 255.255.255.0 10.0.5.1

ip route 10.0.1.0 255.255.255.0 10.0.5.1

ip route 10.0.2.0 255.255.255.0 10.0.5.1

ip route 10.0.3.0 255.255.255.0 10.0.5.1

ip route 10.0.4.0 255.255.255.0 10.0.5.1

ip route 10.0.6.0 255.255.255.0 10.0.5.1

ip route 182.46.0.0 255.255.255.0 10.0.5.1

ip route 153.69.0.0 255.255.255.0 10.0.5.1

ip route 200.10.156.0 255.255.255.0 10.0.5.1

Configure Router 7 static routes

R7 directly knows:

10.0.6.0/24

153.69.0.0/24

Everything else goes to R2: R2 = 10.0.6.1

Configure Router 7 static routes

ip route 10.0.0.0 255.255.255.0 10.0.6.1

ip route 10.0.1.0 255.255.255.0 10.0.6.1

ip route 10.0.2.0 255.255.255.0 10.0.6.1

ip route 10.0.3.0 255.255.255.0 10.0.6.1

ip route 10.0.4.0 255.255.255.0 10.0.6.1

ip route 10.0.5.0 255.255.255.0 10.0.6.1

ip route 182.46.0.0 255.255.255.0 10.0.6.1

ip route 135.82.0.0 255.255.255.0 10.0.6.1

ip route 200.10.156.0 255.255.255.0 10.0.6.1

Configure the servers

Router 1 Server LAN

IP:      10.0.0.10

Mask:    255.255.255.0

Gateway: 10.0.0.1

Router 2 Server LAN

IP:      200.10.156.10

Mask:    255.255.255.0

Gateway: 200.10.156.1

Router 4 Server LAN

IP:      182.46.0.10

Mask:    255.255.255.0

Gateway: 182.46.0.1

Router 6 Server LAN

IP:      135.82.0.10

Mask:    255.255.255.0

Gateway: 135.82.0.1

Router 7 Server LAN

IP:      153.69.0.10

Mask:    255.255.255.0

Gateway: 153.69.0.1

Test step-by-step

Test 1 — R1 to R2

On R1:

ping 10.0.1.2

Test 2 — R2 to R3

On R2:

ping 10.0.2.2

Test 3 — R3 to R4

ping 10.0.3.2

Test 4 — R3 to R5

ping 10.0.4.2

Test 5 — R5 to R6

ping 10.0.5.2

Test 6 — R2 to R7

ping 10.0.6.2

Now test the servers

From Server 0: ping 10.0.0.1

First confirm its gateway.

Then: ping 10.0.1.2

Then: ping 10.0.2.2

Finally test the remote server: ping 153.69.0.10

You should get:Reply from 153.69.0.10: bytes=32 time<1ms TTL=...

Test the opposite direction

From Server 8:

ping 153.69.0.1

Then:

ping 10.0.0.10

If both directions work:

10.0.0.10  ←→  153.69.0.10

your routing is working.

Configure static routes on both sides. A packet needs a route to go forward and a route to come back.

Result from 10.0.0.10 to All the branches

Result from All the branches to 10.0.0.10