Cisco Packet Tracer DHCP Relay project

Phase 1: Configure IP Addresses on All Router Interfaces

R1

conf t

int eth1/0

ip address 172.50.0.1 255.255.255.0

no shut

int eth1/1

ip address 1.0.0.1 255.255.255.0

no shut

R2

conf t

int eth1/0

ip address 12.0.0.1 255.255.255.0

no shut

int eth1/1

ip address 62.0.0.1 255.255.255.0

no shut

int eth1/2

ip address 172.50.0.2 255.255.255.0

no shut

int eth1/3

ip address 222.215.0.1 255.255.255.0

no shut

R3

conf t

int eth1/0

ip address 29.0.0.1 255.255.255.0

no shut

int eth1/1

ip address 12.0.0.2 255.255.255.0

no shut

R4

conf t

int eth1/0

ip address 222.215.0.2 255.255.255.0

no shut

int eth1/1

ip address 129.23.0.1 255.255.255.0

no shut

R5

conf t

int eth1/0

ip address 62.0.0.2 255.255.255.0

no shut

int eth1/1

ip address 212.25.0.1 255.255.255.0

no shut

Phase 2: Verify Directly Connected Networks

On each router:

show ip interface brief

show ip route

You should see only C (Connected) routes.

Example on R1:

R1#show ip route

C 172.50.0.0/24 is directly connected, Eth1/0

C 1.0.0.0/24 is directly connected, Eth1/1

Phase 3: Configure Static Routes

R1

ip route 29.0.0.0 255.255.255.0 172.50.0.2

ip route 129.23.0.0 255.255.255.0 172.50.0.2

ip route 212.25.0.0 255.255.255.0 172.50.0.2

R3

ip route 1.0.0.0 255.255.255.0 12.0.0.1

ip route 129.23.0.0 255.255.255.0 12.0.0.1

ip route 212.25.0.0 255.255.255.0 12.0.0.1

R4

ip route 1.0.0.0 255.255.255.0 222.215.0.1

ip route 29.0.0.0 255.255.255.0 222.215.0.1

ip route 212.25.0.0 255.255.255.0 222.215.0.1

R5

ip route 1.0.0.0 255.255.255.0 62.0.0.1

ip route 29.0.0.0 255.255.255.0 62.0.0.1

ip route 129.23.0.0 255.255.255.0 62.0.0.1

R2 (Core Router)

ip route 1.0.0.0 255.255.255.0 172.50.0.1

ip route 29.0.0.0 255.255.255.0 12.0.0.2

ip route 129.23.0.0 255.255.255.0 222.215.0.2

ip route 212.25.0.0 255.255.255.0 62.0.0.2

Phase 4: Test Routing Before DHCP

Assign temporary static IPs to the PCs:

PC0 (HR)

IP: 29.0.0.10

Mask: 255.255.255.0

Gateway: 29.0.0.1

PC1 (PRODUCTION)

IP: 212.25.0.10

Mask: 255.255.255.0

Gateway: 212.25.0.1

PC2 (SALES)

IP: 1.0.0.10

Mask: 255.255.255.0

Gateway: 1.0.0.1

PC3 (MGMT)

IP: 129.23.0.10

Mask: 255.255.255.0

Gateway: 129.23.0.1

Now test:

PC0 > ping 1.0.0.10

PC0 > ping 212.25.0.10

PC0 > ping 129.23.0.10

All pings should succeed.

Phase 5 – Configure DHCP Server on R2 and DHCP Relay

Once all PCs can ping each other using static IP addresses, proceed with DHCP.

Step 1: Configure R2 as DHCP Server

Exclude Gateway Addresses

R2(config)#ip dhcp excluded-address 1.0.0.1

R2(config)#ip dhcp excluded-address 29.0.0.1

R2(config)#ip dhcp excluded-address 129.23.0.1

R2(config)#ip dhcp excluded-address 212.25.0.1

Step 2: Create DHCP Pool for SALES Network

Network: 1.0.0.0/24

R2(config)#ip dhcp pool SALES

R2(dhcp-config)#network 1.0.0.0 255.255.255.0

R2(dhcp-config)#default-router 1.0.0.1

R2(dhcp-config)#dns-server 8.8.8.8

R2(dhcp-config)#exit

Step 3: Create DHCP Pool for HR Network

Network: 29.0.0.0/24

R2(config)#ip dhcp pool HR

R2(dhcp-config)#network 29.0.0.0 255.255.255.0

R2(dhcp-config)#default-router 29.0.0.1

R2(dhcp-config)#dns-server 8.8.8.8

R2(dhcp-config)#exit

Step 4: Create DHCP Pool for MGMT Network

Network: 129.23.0.0/24

R2(config)#ip dhcp pool MGMT

R2(dhcp-config)#network 129.23.0.0 255.255.255.0

R2(dhcp-config)#default-router 129.23.0.1

R2(dhcp-config)#dns-server 8.8.8.8

R2(dhcp-config)#exit

Step 5: Create DHCP Pool for PRODUCTION Network

Network: 212.25.0.0/24

R2(config)#ip dhcp pool PRODUCTION

R2(dhcp-config)#network 212.25.0.0 255.255.255.0

R2(dhcp-config)#default-router 212.25.0.1

R2(dhcp-config)#dns-server 8.8.8.8

R2(dhcp-config)#exit

Step 6: Verify DHCP Pools

R2#show ip dhcp pool

Expected output:

Pool SALES

Pool HR

Pool MGMT

Pool PRODUCTION

Step 7: Configure DHCP Relay on R1

The helper address must point to the DHCP Server (R2).

R1(config)#interface eth1/1

R1(config-if)#ip helper-address 172.50.0.2

Step 8: Configure DHCP Relay on R3

R3(config)#interface eth1/0

R3(config-if)#ip helper-address 12.0.0.1

Step 9: Configure DHCP Relay on R4

R4(config)#interface eth1/1

R4(config-if)#ip helper-address 222.215.0.1

Step 10: Configure DHCP Relay on R5

R5(config)#interface eth1/1

R5(config-if)#ip helper-address 62.0.0.1

Important: Configure ip helper-address on the LAN-facing interface connected to the PCs, not on the WAN interface.

Step 11: Change PCs to DHCP

For each PC:

Desktop

→ IP Configuration

→ DHCP

Wait a few seconds.

Step 12: Verify DHCP Leases on R2

R2#show ip dhcp binding

Example:

IP address      Client-ID

1.0.0.2

29.0.0.2

129.23.0.2

212.25.0.2

Step 13: Verify from PCs

On each PC:

ipconfig

You should receive:

| PC | Expected IP |
| --- | --- |
| PC0 (HR) | 29.0.0.x |
| PC1 (PRODUCTION) | 212.25.0.x |
| PC2 (SALES) | 1.0.0.x |
| PC3 (MGMT) | 129.23.0.x |

Step 14: Final Testing

From PC0:

ping 1.0.0.x

ping 212.25.0.x

ping 129.23.0.x

From PC2:

ping 29.0.0.x

ping 212.25.0.x

ping 129.23.0.x

If all pings succeed, your Static Routing + DHCP Relay project is complete.